Part 1: Multiple-choice questions (24 questions, 6 scores)

Q.1 (G2.2, G6.2):  What is the characteristic of the Von Neumann computer architecture?

| - A common memory is used for both program and data | - Program memory and data memory are separate |
| --- | --- |
| - A simple computer architecture | - A complex computer architecture |

Q.2 (G2.2, G6.2): Which statement is CORRECT about 8051 MCU?

| - Including 128B internal ROM and 4KB internal RAM | - Not allowed to use external memory |
| --- | --- |
| - Having two 16-bit timers | - Address bus of 20 bits |

Q.3 (G2.2, G6.2): In 8051 MCU, which of the following registers is NOT a bit addressable register?

| - TCON | - PSW |
| --- | --- |
| - TLx, THx | - SBUF |

Q.4 (G2.2, G6.2): What is the address of Timer 1 Interrupt vector in 8051 MCU?

| - 0003h | - 0013h |
| --- | --- |
| - 0023h | - 000Bh |

Q.5 (G2.2, G6.2): What is the width of 8051 address bus?

| - 16 bit | - 20 bit |
| --- | --- |
| - 32 bit | - None of the choices above |

Q.6 (G4.1, G6.2): Given a source code for 8051 MCU:

MOV A, #45H

MOV B, #F5H

MUL AB

What are the values of registers A and B after processing the codes above?

| - A = 09H, B = 42H | - A = 45H, B = F5H |
| --- | --- |
| - A = F5H, B = 45F | - A = 42H, 09H |

Q.7 (G2.2, G6.2): Register PSW gets value of 98 in decimal base. What register bank is used now in 8051 MCU?

| - Bank 0 | - Bank 1 |
| --- | --- |
| - Bank 2 | - Bank 3 |

Q.8 (G4.1, G6.2): Given a source code for 8051 MCU:

MOV A, #E8H

MOV B, #1BH

DIV AB

What are the values of registers A and B after processing the codes above?

| - A = 08H, B =10H | - A = 08H, B =F5H |
| --- | --- |
| - A = 10H, B =08H | - A = 45H, B =10H |

Q.9 (G2.2, G6.2): What is the correct value configed to SCON register to allow 8051 MCU to receive data from UART:

| - 10H | - 00H |
| --- | --- |
| - EFH | - 80H |

Q.10 (G2.2, G6.2): Which pins of 8051 are used for serial connection (Rx, Tx)?

| - P3.0 and P3.1 | - P3.2 and P3.3 |
| --- | --- |
| - P3.4 and P3.5 | - P3.6 and P3.7 |

Q.11(G4.1, G6.2): Given a OSC. Clock XTAL = 6 MHz to 8051 MCU. What is the machine cycle of the MCU?

| - 1 μs | - 1.5 μs |
| --- | --- |
| - 2 μs | - None |

Q.12 (G2.2, G6.2): What addressing mode is used to access the SFR of 8051 MCU?

| - Immediate addressing mode | - Register addressing mode |
| --- | --- |
| - Direct addressing mode | - Indirect addressing mode |

Q.13 (G2.2, G6.2): Register IE of 8051 MCU is configured with a value of 8AH in decimal base. What interrupts are used now?

| - INT1, Timer 0 | - INT1, Timer 1 |
| --- | --- |
| - Timer 0, Timer 1 | - No interrupts are allowed |

Q.14 (G4.1, G6.2): Given a source code for 8051 MCU:

MOV		DPTR, #4000h

MOV		A, #5

MOVC  	A, @A + DPTR

What addressing mode is used at line MOVC  A, @A + DPTR?

| - Immediate addressing mode | - Register addressing mode |
| --- | --- |
| - Direct addressing mode | - Indirect addressing mode |

Q.15 (G2.2, G6.2): How many operands are there at most in an 8051 ASM instruction?

| - 1 | - 2 |
| --- | --- |
| - 3 | - 4 |

Q.16 (G2.2, G6.2): What is the meaning of the instruction “COUNT EQU 30” on 8051 MCU:

| - Defining a constant COUNT with value of 30 | - Defining a varaiable COUNT with the initial value of 30 |
| --- | --- |
| - Defining a string | - Defining an array of 30 elements |

Q.17 (G2.2, G6.2): What is the CORRECT definition in 8051 ASM codes?

| - 30Shine EQU 30 | - Count DB 30 |
| --- | --- |
| - DB Shine30 30 | - Count = 30 |

Q.18 (G2.2, G6.2): Assume that a 8086 MPU references to a logic address of 1320:0018H. What is the physical address corresponding to that logic address?

| - 1A4D8 KB | - 13218H |
| --- | --- |
| - 13380 KB | - Unknown |

Q.19 (G4.1, G6.2): Given a source code for 8086 MPU:

MOV AX, #F5H

MOV BX, #45H

DIV BX

What are the quotient and remainder of the division and where are they stored?

| - Quotient of 03H stored at AX, remainder of 26H stored at DX | - Quotient of 03H stored at AX, remainder of 00H stored at BX |
| --- | --- |
| - Quotient of 03H stored at AX, remainder of 26H stored at BX | - Quotient of 03H stored at CX, remainder of 26H stored at DX |

Q.20 (G4.1, G6.2): Given the configuration of Timer 1 of 8051 MCU is TMOD = 10H, TL1 = 38H, TH1 = FFH. The input clock is 6 MHz. How long is TF1 flag set to 1 after the timer starts?

| - 100 μs | - 200 μs |
| --- | --- |
| - 300 μs | - 400 μs |

Q.21 (G4.1, G6.2): Assume that a 8086 MPU has segment register CS = 0300H and DS = 0100H. Given the following source codes:

MOV AX, #F5H

MOV DI, #05H

MOV [DI]+05H, AX

What is the physical address where the value F5H is stored after finishing the codes above?

| - 0100AH | - 01013H |
| --- | --- |
| - 01005H | - 00013H |

Q.22 (G4.1, G6.2): Assume that a clock of 12 MHz is provided to 8051 MCU. How many MOV instructions are done for 1s if one MOV instruction requests one machine cycle?

| - 1.000 | - 500.000 |
| --- | --- |
| - 5.000 | - 1.000.000 |

Q.23 (G2.2, G6.2): What is the function of interrupt vector table?

| - Storing the interrupt service routine for interrupt requests | - Storing the address of the interrupt service routine for interrupt requests |
| --- | --- |
| - a and b are correct | - a and b are incorrect |

Q.24 (G2.2, G6.2): In 8086 MPU, which component generates the address of instructions?

| - Bus Interface Unit | - Logic Control |
| --- | --- |
| - ALU | - Instruction pointer |

Part 2: Responding question (4 scores) (G4.1, G6.2)

Compose an ASM program for 8051 using timer to generate a clock with the cycle of 1ms and duty cycle of 80% at pin P2.6. Assume that the provided clock is 16MHz.